FIT3143 PARALLEL COMPUTING

Applied #1 Assessment - Week 6

Distributed Wireless Sensor Network
Electric Vehicle Charging Navigation System Simulation

TEAM MEMBERS

Savin Vindiv De Alwis

Student ID: 35221631 | sdea0018@student.monash.edu

Shamle Thilaksiri

Student ID: 35512075 | wthi0003@student.monash.edu

Contents

| Executive Summary | 3 |  |
| --- | --- | --- |
| 1. Task 1 - Network Topology | 4 |  |
| 1.1 Requirements and assumptions | 4 |
| 1.2 Topology comparison criteria | 4 |
| 1.3 Comparison of Week 5 topologies | 4 |
| 1.4 Selected topology: 2-D mesh | 5 |
| 2. Task 2 - Architecture Design | 6 |  |
| 2.1 Hybrid MIMD simulation architecture | 6 |
| 2.2 Static software structure | 8 |
| 2.3 Simulation workflow and message passing | 9 |
| 2.4 Message protocol and synchronization | 10 |
| 2.5 Base-station selection algorithm | 10 |
| 2.6 Machines, cores and bandwidth | 10 |
| 3. Task 3 - Communication Analysis | 12 |  |
| 3.1 Analytical model and message sizes | 12 |
| 3.2 Scaling with number of charging nodes | 12 |
| 3.3 Scaling with number of base stations | 14 |
| 3.4 Additional sensitivity analysis | 15 |
| 3.5 Communication-analysis conclusions | 16 |
| Conclusion, Limitations and Future Work | 17 |  |
| References | 18 |  |
| AI Declaration | 19 |  |

# Executive Summary

Simulates the EVCNS wireless sensor network defined in Applied #1.

Uses a 2-D mesh for geographically adjacent EV charging stations.

Uses MPI processes for distributed charging nodes and the base station.

Uses OpenMP threads inside each charging-node process for shared-memory parallelism.

Implements the hybrid MIMD model encouraged by the HD rubric and supported by unit theory [1]-[5].

2-D mesh: maximum four orthogonal neighbours, O(N) link cost, and direct immediate-neighbour communication.

MPI rank 0 represents the base station; ranks 1..N represent charging nodes.

Each node stores charging-port states and aggregates utilisation with an OpenMP parallel-for reduction.

Over-threshold nodes query neighbours with non-blocking MPI.

A congestion alert is sent only when all valid neighbours are also above threshold.

Transmission model: T = (L x H) / B on 1 Gbit/s links [3].

Fixed-size one-hop message transmission time remains O(1) as N grows.

Aggregate communication demand grows O(N).

With S balanced base stations, base-station service demand falls toward O(N/S), while single-message serialization is unchanged.

Additional analysis covers reporting frequency; queueing, wireless effects, and server synchronization are outside the baseline model.

# 1. Task 1 - Network Topology

Charging nodes are distributed across a 2-D geographic area and communicate with immediate adjacent nodes.

Week 5 comparison metrics: node degree, diameter, bisection width, arc connectivity, and link cost.

Latency and bandwidth are key MIMD fabric-performance considerations [3].

The selected topology must balance communication performance with WSN realism.

## 1.1 Requirements and assumptions

| ID | Assumption used by the simulator |
| --- | --- |
| A1 | For analytical graphs, N is taken as a perfect square and the EVCS network is a sqrt(N) x sqrt(N) Cartesian grid. |
| A2 | Charging-node adjacency is orthogonal only (north, south, east and west). Diagonal stations are not direct neighbours. |
| A3 | Corner nodes have degree 2, edge nodes degree 3 and interior nodes degree 4; therefore d <= 4. |
| A4 | Every charging-node process can communicate with the base-station process over the cluster interconnect; neighbour messages are one logical mesh hop. |
| A5 | The utilization threshold is a configurable simulation parameter. No numerical threshold is invented because the assessment specification does not provide one. |

Table 1. Simulation assumptions used to resolve details not fixed by the assessment specification.

## 1.2 Topology comparison criteria

Node degree: number of I/O links connected to a node; indicates local communication fan-out.

Network diameter: maximum hop count between any pair of nodes; bounds worst-case path length.

Bisection width: minimum links cut to split the network approximately in half; relates to aggregate bandwidth.

Arc connectivity: links whose removal can disconnect the network; supports fault-tolerance analysis.

Cost: total number of communication links [3].

Application-specific priorities: geographic realism and immediate-neighbour communication.

## 1.3 Comparison of Week 5 topologies

| Topology | Node degree | Diameter | Bisection width | Arc connectivity | Cost |
| --- | --- | --- | --- | --- | --- |
| Linear array | 1 or 2 | N - 1 | 1 | 1 | N - 1 |
| Ring | 2 | N/2 | 2 | 2 | N |
| Star | 1 or N - 1 | 2 | 1 | 1 | N - 1 |
| Binary tree | 1, 2 or 3 | 2 log2((N+1)/2) | 1 | 1 | N - 1 |
| 2-D mesh | 2, 3 or 4 | 2(sqrt(N)-1) | sqrt(N) | 2 | 2(N-sqrt(N)) |
| 2-D wraparound / torus | 4 | sqrt(N) | 2sqrt(N) | 4 | 2N |
| 3-D cube | 3, 4, 5 or 6 | 3(cuberoot(N)-1) | N^(2/3) | 3 | 3(N-N^(2/3)) |
| Hypercube | log2(N) | log2(N) | N/2 | log2(N) | N log2(N)/2 |
| Completely connected | N - 1 | 1 | N^2/4 | N - 1 | N(N-1)/2 |

Table 2. Static interconnection-network parameters taught in Week 5.

Source: FIT3143 Topic 5, "Summary of Static Network (Fabric) Parameters" [3].

Week 5 duplicate slides conflict on 2-D mesh arc connectivity and 3-D cube cost; Table 2 uses Topic 5 summary values: mesh arc connectivity = 2 and 3-D cube cost = 3(N-N^(2/3)).

| Topology | Geographic realism | Immediate-neighbour fit | Scalability / resilience | Assessment suitability |
| --- | --- | --- | --- | --- |
| Linear array | Poor | Limited to two neighbours | O(N) links but O(N) diameter | Low - one-dimensional and long paths |
| Ring | Poor | Two neighbours | O(N) links; better resilience than a line | Low - wraparound is geographically artificial |
| Star | Moderate | Traffic is centre-mediated | O(N) links; central bottleneck/failure point | Low-Medium - poor neighbour realism |
| Binary tree | Poor | Hierarchy, not physical adjacency | O(N) links; O(log N) depth | Low - topology does not match a 2-D road/grid area |
| 2-D mesh | Excellent | Direct orthogonal neighbours | O(N) links; O(sqrt(N)) diameter | Best - direct match to specification |
| 2-D torus | Moderate | Four neighbours | O(N) links; lower diameter than mesh | Medium-High - opposite edges become neighbours |
| 3-D cube | Poor | Includes unnecessary third dimension | O(N) links; good connectivity | Medium - mathematically viable, physically unrealistic |
| Hypercube | Poor | Logical rather than geographic neighbours | O(N log N) links; O(log N) diameter | Medium - fast but weak WSN realism |
| Complete | Poor | Every node is a neighbour | O(N^2) links; diameter 1 | Low - excessive links and unrealistic communication |

Table 3. Application-specific comparison of all Week 5 topologies.

## 1.4 Selected topology: 2-D mesh

Figure 1. Selected 3 x 3 logical WSN. Solid lines are orthogonal neighbour links; dashed lines illustrate node-to-base-station MPI communication.

2-D mesh directly models orthogonal geographic neighbours.

The specification requires immediate-adjacent node communication [1].

Neighbour relationships are native to the topology rather than imposed through a hub or arbitrary logical mapping.

Bounded node degree d <= 4 keeps neighbour fan-out small.

Mesh link cost is 2(N - sqrt(N)) = O(N), versus O(N^2) for a completely connected network.

Mesh diameter is 2(sqrt(N)-1) = O(sqrt(N)), which is worse than a hypercube.

This trade-off is acceptable because the workload is local and geographic realism is the priority.

Grid coordinates support Manhattan distance d = |r_s-r_j| + |c_s-c_j| for nearest-node redirection.

Manhattan distance matches the minimum number of orthogonal grid steps.

Real GPS or road-network distance can replace this metric without changing the MPI architecture.

# 2. Task 2 - Architecture Design

Target platform: a cluster of multi-core machines.

Distributed-memory MIMD uses separate processor/memory pairs connected by an interconnection network.

Shared-memory MIMD uses multiple processors over a shared address space.

Hybrid MIMD combines both models [3].

MPI provides process-level distributed parallelism; OpenMP provides thread-level shared-memory parallelism.

## 2.1 Hybrid MIMD simulation architecture

MPI rank 0 models the base station; ranks 1..N each model one EV charging station (EVCS).

Each EVCS owns its local state and P ChargingPort objects.

OpenMP threads update ports and aggregate utilisation concurrently.

Avoiding one MPI process per port reduces unnecessary distributed-memory overhead.

Parallelism follows the hardware hierarchy: MPI across processes/machines, OpenMP within a process.

Figure 2. Hybrid deployment example for the 3 x 3 WSN: 10 MPI ranks across three 16-core machines, with four OpenMP threads per EVCS process.

Figure 2 shows one valid deployment; physical MPI rank placement is not fixed.

Rank-to-grid coordinates preserve the logical WSN regardless of machine placement.

Task 3 assumes 1 Gbit/s inter-machine links.

Even if colocated ranks use faster shared-memory transport, the analysis retains 1 Gbit/s for consistency and conservatism.

## 2.2 Static software structure

Figure 3. UML-style static structure of the simulation model.

| Component | Responsibility |
| --- | --- |
| BaseStation | Rank 0; maintains NodeStatus table, logs updates, handles congestion alerts, finds closest available node, sends redirection commands. |
| EVChargingNode | One object/process per station; owns coordinates, threshold, neighbour ranks and local ports; performs status, neighbour and alert logic. |
| ChargingPort | Local shared-memory object representing one free/occupied charging port. Port work is partitioned among OpenMP threads. |
| NodeStatus | Compact status record sent to and stored by the base station. It contains node ID, grid position, free/total ports and utilisation. |
| MessageTags | Semantic MPI tags that separate status, neighbour-query, neighbour-reply, congestion-alert and redirect traffic. |

Table 4. Simulation components and responsibilities.

## 2.3 Simulation workflow and message passing

Each EVCS process runs an event loop.

An OpenMP parallel-for scans or updates the local port array.

OpenMP reduction safely combines occupied-port counts without a shared-update race or critical-section serialization [5].

The node calculates utilisation and sends NodeStatus to the base station.

If utilisation exceeds the threshold, query every valid mesh neighbour.

Use MPI_Isend/MPI_Irecv to overlap independent neighbour exchanges instead of serial blocking.

Post receives, issue queries, then synchronize locally with MPI_Waitall [4].

Send CONGESTION_ALERT to rank 0 only when all neighbour replies are also above threshold.

Figure 4. Sequence diagram for one simulation interval and the overload path.

No global MPI_Barrier is used for each interval.

A global barrier would force fast ranks to wait for the slowest rank.

Synchronization is local: a node waits only for neighbour replies required for its decision.

Message passing therefore provides both data movement and synchronization [3].

## 2.4 Message protocol and synchronization

| Tag | Direction | Payload fields | Bytes* | MPI behaviour |
| --- | --- | --- | --- | --- |
| STATUS_UPDATE | EVCS -> base | NodeStatus: nodeID, row, column, freePorts, totalPorts, utilisation | 28 | MPI_Isend; base receives asynchronously |
| NEIGHBOUR_QUERY | EVCS -> adjacent EVCS | requesterID, simulationRound | 8 | MPI_Isend after posting reply receives |
| NEIGHBOUR_REPLY | Adjacent EVCS -> requester | nodeID, freePorts, totalPorts, utilisation | 20 | MPI_Isend / requester MPI_Irecv + MPI_Waitall |
| CONGESTION_ALERT | EVCS -> base | NodeStatus plus simulationRound | 32 | MPI_Isend; event-driven at base station |
| REDIRECT_COMMAND | Base -> EVCS | targetNodeID, targetRow, targetColumn, simulationRound | 16 | Point-to-point response to alerting node |

Table 5. Message-passing protocol used by the architecture.

Payload sizes assume 4-byte int and 8-byte double fields.

Sizes exclude MPI/transport headers and implementation-specific padding.

Use an MPI derived datatype or packed representation to send heterogeneous NodeStatus fields as one logical message.

Post reply receives before queries; use non-blocking operations and MPI_Waitall.

This ordering avoids circular blocking send/receive deadlock.

The base station can use non-blocking receive/probe with MPI_ANY_SOURCE and message tags.

OpenMP reduction synchronizes only local shared-memory work; no cross-process shared state is used.

## 2.5 Base-station selection algorithm

The base station stores the latest status for every EVCS rank.

On CONGESTION_ALERT from source i, keep only candidates with a free port and utilisation at or below the threshold.

For Cartesian grid coordinates, compute Manhattan distance from the source to every candidate.

| d(i,j) = ¦r_i - r_j¦ + ¦c_i - c_j¦ | (1) |
| --- | --- |

Select the candidate with minimum distance d.

Resolve ties using the lowest MPI rank for deterministic, reproducible results.

Send REDIRECT_COMMAND with the selected node ID and coordinates.

Baseline search cost is O(N) per alert.

For very large N, spatial indexing or regional base stations could reduce search cost.

## 2.6 Machines, cores and bandwidth

N = charging nodes; P = ports per node; T = OpenMP threads per EVCS; C = usable physical cores per machine.

Use T = min(P, T_max), not one thread per port.

MPI processes required: N + 1.

Approximate physical-core demand: NT + 1 when rank 0 uses one core.

Bind cores to prevent OpenMP oversubscription.

| MPI processes = N + 1 | (2) |
| --- | --- |
| Required cores = N T + 1 | (3) |
| Machines = ceil((N T + 1) / C) | (4) |

3 x 3 example: N = 9, T = 4, C = 16.

Requires 10 MPI ranks, 37 cores, and 3 machines.

Figure 2 placement: machine 1 hosts rank 0 plus EVCS ranks 1-3 (13 cores); machines 2 and 3 host three EVCS ranks each (12 cores).

Square-mesh undirected links: E = 2(N - sqrt(N)).

Directed neighbour relationships: 2E = 4(N - sqrt(N)).

With f simulation intervals per second, worst-case payload bandwidth sums all five message classes per interval:

| BW_payload = f [N L_status + 4(N-sqrt(N))(L_query + L_reply) + N(L_alert + L_redirect)] x 8 | (5) |
| --- | --- |

At N = 256 and f = 10 Hz: 46,336 bytes/interval, or approximately 3.707 Mbit/s total payload traffic.

Base-station-bound payload (status + alert + redirect) is approximately 1.556 Mbit/s.

Both are far below 1 Gbit/s under the payload-only model.

Headers, queueing, processing, contention, and propagation are excluded here and addressed in Task 3.

# 3. Task 3 - Communication Analysis

Task 3 analyses every message type on 1 Gbit/s links as charging-node and base-station counts change.

Separate per-message transmission time from aggregate serialization/service demand.

Increasing N does not change one fixed message's bit length, but it increases the number of communicating processes.

## 3.1 Analytical model and message sizes

Week 5 store-and-forward model: transmission latency is proportional to message length and hop count.

| T_tx = (L x H) / B | (6) |
| --- | --- |

L = message length (bits), H = hop count, and B = channel bandwidth (bits/s) [3].

For B = 10^9 bit/s and baseline one-hop traffic, H = 1.

Neighbour traffic is one mesh hop; node-to-base traffic is treated as one cluster-link transmission.

Queueing, processing, and propagation delays are excluded to match the Week 5 transmission model.

| Message | Payload bytes | Bits | One-hop T_tx (us) | Per-message vs N |
| --- | --- | --- | --- | --- |
| STATUS_UPDATE | 28 | 224 | 0.224 | O(1) |
| NEIGHBOUR_QUERY | 8 | 64 | 0.064 | O(1) |
| NEIGHBOUR_REPLY | 20 | 160 | 0.160 | O(1) |
| CONGESTION_ALERT | 32 | 256 | 0.256 | O(1) |
| REDIRECT_COMMAND | 16 | 128 | 0.128 | O(1) |

Table 6. One-hop payload transmission time for every message type on a 1 Gbit/s link.

## 3.2 Scaling with number of charging nodes

For fixed-size one-hop messages, L, H, and B do not depend on N.

Per-message payload transmission time therefore remains O(1).

Figure 5 intentionally shows flat curves; N increases message count, not individual message length.

Figure 5. Per-message transmission time versus N. Fixed payload size and H = 1 make each message O(1).

Square-mesh undirected links: E = 2(N - sqrt(N)).

Worst-case directed query/reply relationships: 2E = 4(N - sqrt(N)).

Neighbour query/reply counts therefore grow O(N).

Periodic status, worst-case alerts, and worst-case redirects also grow O(N).

Aggregate payload serialization demand:

| T_status,total = N L_status / B = O(N) | (7) |
| --- | --- |
| T_query,total = 4(N-sqrt(N)) L_query / B = O(N) | (8) |
| T_reply,total = 4(N-sqrt(N)) L_reply / B = O(N) | (9) |
| T_alert,total = N L_alert / B = O(N) | (10) |
| T_redirect,total = N L_redirect / B = O(N) | (11) |

Figure 6. Worst-case aggregate payload serialization time per simulation round as N increases. All message classes grow linearly in the large-N limit.

More nodes do not make one neighbour message slower in the baseline model.

More active MPI ranks increase aggregate communication and contention potential.

A single base station receives O(N) status/alert traffic per interval and is the main fan-in point.

Mesh neighbour scalability remains strong because d <= 4.

If base-station traffic were routed through the mesh, worst-case distance would be D = 2(sqrt(N)-1) = O(sqrt(N)).

Store-and-forward worst-case per-message transmission would then scale O(sqrt(N)).

The baseline uses direct logical node-to-base communication, as allowed by the assessment.

Multi-hop routing is therefore a future scalability consideration, not the baseline model.

## 3.3 Scaling with number of base stations

Adding base stations does not change L/B for one message on a 1 Gbit/s link.

It does not affect EVCS-neighbour query/reply traffic.

Its benefit is load distribution.

With S balanced base stations, each handles approximately N/S nodes; base-station-bound service demand becomes O(N/S).

| T_base,peak approximately (N/S) L / B = O(N/S) | (12) |
| --- | --- |

Figure 7. Effect of increasing base stations for N = 256. Base-station-bound endpoint service demand decreases approximately as 1/S; neighbour query/reply demand is unchanged.

Figure 7 shows peak endpoint serialization demand per interval for balanced assignment.

STATUS_UPDATE, CONGESTION_ALERT, and REDIRECT_COMMAND decrease as S grows.

NEIGHBOUR_QUERY and NEIGHBOUR_REPLY remain constant because EVCS degree stays <= 4.

Real systems may also reduce queueing and processing contention, but those effects are outside the transmission equation.

Base-station synchronization or partitioning adds coordination overhead and can reduce ideal 1/S scaling.

## 3.4 Additional sensitivity analysis

HD analysis includes a third scaling factor beyond N and S [2].

Reporting/simulation frequency f is a direct communication driver.

With fixed payload per interval, traffic scales linearly as O(f).

| BW_payload(f) = f x Bytes_per_round x 8 = O(f) | (13) |
| --- | --- |

Figure 8. Payload-bandwidth sensitivity to simulation/reporting frequency for N = 256 under the worst-case message mix.

At N = 256, worst-case payload is 46,336 bytes per interval.

Traffic is approximately 0.371 Mbit/s at 1 Hz, 3.707 Mbit/s at 10 Hz, and 7.414 Mbit/s at 20 Hz.

Even 20 Hz remains below 1% of a 1 Gbit/s link in the payload-only model.

Observed latency may be higher because of headers, MPI protocol behaviour, wireless interference, and simultaneous-flow queueing.

## 3.5 Communication-analysis conclusions

| Quantity | Complexity | Reason |
| --- | --- | --- |
| Per-message neighbour traffic | O(1) | One hop, fixed payload; bounded node degree. |
| Per-message node/base traffic | O(1) | Direct logical MPI channel in baseline architecture. |
| Aggregate status traffic | O(N) | Every EVCS reports once per interval. |
| Aggregate worst-case neighbour traffic | O(N) | 4(N-sqrt(N)) directed neighbour relationships. |
| Aggregate worst-case alerts/redirects | O(N) | At most one alert/redirect pair per active node per interval in the model. |
| Peak base-station load with S servers | O(N/S) | Assumes even partitioning and independent ingress capacity. |
| Reporting-frequency sensitivity | O(f) | Payload volume per interval is constant; intervals per second increase. |

Table 7. Communication-complexity conclusions for the proposed architecture.

# Conclusion, Limitations and Future Work

2-D mesh best matches immediate geographic adjacency, bounded degree, and O(N) link cost.

Hybrid MIMD maps WSN components to MPI ranks and port-level work to OpenMP threads.

Non-blocking neighbour communication overlaps independent exchanges.

Local synchronization avoids unnecessary global barriers.

Fixed-size one-hop messages have O(1) transmission time; aggregate traffic grows O(N).

A single base station is the main fan-in bottleneck.

With S balanced base stations, peak server demand approaches O(N/S), while single-message serialization is unchanged.

At N = 256 and 20 Hz, the modelled worst-case payload remains below 1% of 1 Gbit/s.

Baseline abstractions exclude radio interference, packet loss, failure routing, propagation, MPI headers, queueing, OS scheduling, and detailed road travel distance.

The base station is modelled as a direct MPI endpoint rather than a multi-hop wireless destination.

These assumptions keep Task 3 aligned with the specified 1 Gbit/s transmission model.

Future work: regional base stations, server-status replication, failure-aware routing, measured MPI latency/throughput, GPS/road distance, and dynamic vehicle arrivals.

# References

[1] Monash University, FIT3143 Applied #1 Assessment (Week 6): Distributed Wireless Sensor Network, 2026.

[2] Monash University, FIT3143 Applied #1 Marking Rubric (Week 6), 2026.

[3] FIT3143 Parallel Computing, Topic 5: Multiple Instruction Multiple Data and Distributed Memory Architectures, Monash University, 2024. Relevant material: distributed/shared/hybrid MIMD, interconnection-network parameters, topology table, switching, and latency.

[4] FIT3143 Week 5 Applied Preparation / MPI material, Monash University, 2024. Relevant material: MPI_Send/MPI_Recv, MPI_Isend/MPI_Irecv, MPI_Wait/MPI_Test, and non-blocking message passing.

[5] FIT3143 Week 3 Lab Preparation, Monash University, 2024. Relevant material: OpenMP shared-memory data environment, parallel-for work sharing, critical sections, and reduction.

[6] FIT3143 Parallel Computing, Topic 1: Introduction to Parallel Computing, Monash University, 2024. Relevant material: parallel versus distributed computing and MIMD context.

[7] Consolidated MPI examples supplied with the unit/project sources: rank/size, point-to-point communication, custom MPI datatypes, and MPI_Wtime.

# AI Declaration

Generative AI was used during assessment preparation.

ChatGPT (OpenAI) assisted with report structuring/editing, interpretation of supplied assessment/rubric/unit materials, topology and architecture refinement, diagram/graph generation, and communication-formula/terminology checks.

The authors remain responsible for validating technical accuracy and explaining the work without external tools during the presentation and Q&A.

Relevant AI prompt records should be exported and submitted as a separate PDF if required by the assessment declaration process.

Declared tool: ChatGPT (OpenAI).

Purpose: preparation, analysis support, editing, diagram/graph generation, and formatting.

Assessment scope: Tasks 1-3; presentation Q&A excluded.